After Action Review – Lessons Learned

Blue Spruce Structure Fire, November 28, 2009

The following items were identified during an After Action Review conducted December 11, 2009 in response to the question, “What would you do differently?”. The AAR was for a three-alarm, residential structure fire that occurred on November 28 at 695 Blue Spruce Drive, Blackhawk. These “lessons learned” were identified by the personnel involved. Only the first alarm assignment was involved in the AAR, but any other personnel who were involved may submit additional information for inclusion in this report. This AAR will be posted to the District intranet, (iNet), for future review.

Please submit any constructive feedback to the Training Division regarding this incident or process.

- Bring tools necessary for your assignment with you to the fire location, (i.e. pike poles for interior fire attack)

- Secure utilities early, even when there are no obvious signs of hazard, like electrical arcing or gas fires.

- When confronted with a fire apparatus mechanical issue, if the problem cannot be resolved within a few seconds of troubleshooting, immediately utilize available bypasses to ensure operations can safely continue. Immediately communicate through appropriate channels any potential delay to carrying out your assignment.

- In high or extreme wind situations, truck Apparatus Operators should continue to advise Command appropriately when wind speed limits are reached, including advisement of whether continued use will risk equipment damage or constitute a safety hazard. Command should react accordingly to not unnecessarily damage equipment and to never create additional safety hazards for personnel on scene.

- Truck Apparatus Operators should assess the need for personnel on an aerial ladder when flowing water or when operational wind limits are reached so as to not create a safety hazard for personnel on scene or risk unnecessary equipment damage.

- When large or intense fires with wind increase the spotting potential, post lookouts and spot- fire patrols downwind.

- Maintain equipment operational readiness, (i.e. make sure radios are programmed correctly).

- If communication is not clearly understood, make an attempt to clarify the meaning rather than assume, (i.e. determine if Dispatch is “prompting”).

- Ensure that appropriate supervisory depth is utilized at command level, (i.e. Safety Officer).

- Provide for appropriate automatic response for Breathing Support unit on all working structure fires.

- ADDRESSED: BS31 has been added as an automatic response to all 2nd alarm structure fires.